Contestant Number:

Time:

Rank:

COMPUTER NETWORK TECHNOLOGY

(300)

NATIONAL – 2020

TOTAL POINTS		 (100 points)

Failure to adhere to any of the following rules will result in disqualification:

- Contestant must hand in this test booklet and all printouts. Failure to do so will result in disqualification.

- No equipment, supplies, or materials other than those specified for this event are allowed in the testing area.  No previous BPA tests and/or sample tests or facsimile (handwritten, photocopied, or keyed) are allowed in the testing area.

- Electronic devices will be monitored according to ACT standards.

No more than sixty (60) minutes testing time

Property of Business Professionals of America.

May be reproduced only for use in the Business Professionals of America

Workplace Skills Assessment Program competition.

# Choose the best answer to the following questions.  Mark answers clearly if using a scantron form.  Mark A if the statement is true.  Mark B if the statement is false.

- Which of the following does a switch use for increased performance?

  - simplex

  - half duplex

  - full duplex

  - sliding duplex

- What standard describes CSMA/CD?

  - 801.2

  - 802.3

  - 802.5

  - 802.11

- Which layer in the OSI model covers HTTP, FTP, and RDC?

  - Physical

  - Session

  - Application

  - Presentation

- Which layer of the OSI model is used to create a connection so that a host can transfer files?

  - Physical

  - Session

  - Application

  - Presentation

- Which protocol do you use as the transport protocol for a video application?

  - TCP

  - UDP

  - FTP

  - RDC

- What type of address is 202.23.2.255?

  - Multicast address

  - Broadcast address

  - Unicast address

  - Anycast address

- You have a subnet mask of 255.255.240.0. What would the CIDR notation be?

  - /16

  - /20

  - /24

  - /28

- How many bits is an IPv6 address?

  - 32

  - 48

  - 64

  - 128

- What type of address is 2001:4212:0000:34DB:0000:0000:0000:4231?

  - MAC address

  - IPv4

  - IPv6

  - Anycast address

- What protocol does the ping command use to test network connectivity?

  - TCP

  - UDP

  - ICMP

  - FTP

- What command do you use to clear the DNS cache on a local system?

  - ipconfig /registerdns

  - ipconfig /cleardns

  - ipconfig /flushdns

  - ipconfig /releasedns

- What command do you use to test a system’s IPv4 protocol stack?

  - ping 127.0.0.1

  - ping self

  - ping 255.255.255.255

  - ping defaultgateway

- What command do you use to display active TCP or UDP connections?

  - nbtstat

  - netstat

  - ipconfig

  - nslookup

- Which of the following is not a valid traffic form in IPv6?

  - Unicast

  - Link local

  - Anycast

  - Broadcast

- What routing protocol characteristic indicates the believability of the routing protocol (as opposed to other routing protocols)?

  - Weight

  - Metric

  - Administrative distance

  - SPF algorithm

- What command is used to display NetBIOS over TCP/IP statistics?

  - nbtstat

  - netstat

  - ipconfig

  - nslookup

- What is the admin distance for RIP?

  - 90

  - 120

  - 110

  - 170

- What command clears the NetBIOS name cache table?

  - nbtstat -r

  - nbtstat -R

  - nbtstat -RR

  - nbtstat -s

- What command tests network connectivity while showing you each hop along the way and the degree of packet loss?

  - ping

  - pathping

  - nbtstat

  - tracert

- Which of these ports does DHCP use?

  - 80

  - 67

  - 23

  - 500

- You have a computer with an address of 169.254.32.23 and a subnet mask of 255.255.0.0, yet you cannot connect to your local file servers. What is most likely the problem?

  - It cannot communicate with a DHCP server.

  - The DNS servers specified are incorrect or are down

  - NetBIOS over TCP/IP has not been enabled

  - The network card is connected properly to the network

- What name does Windows Server 2008 R2 use for Terminal Services?

  - MS Telnet Server

  - MS Primary Services

  - MS Thin Server

  - Remote Desktop Services

- What port does Remote Desktop Services use?

  - 443

  - 501

  - 389

  - 3389

- What protocol would you use to create VPN tunnels so that users can connect to your internal network while at home using Windows Server 2008 R2?

  - Microsoft RASA

  - Microsoft RRAS

  - Microsoft RDB

  - Microsoft VNP

- Which component of IPSec generates the encryption and authentication keys?

  - SA

  - AH

  - ESP

  - MPPE

- What does X.25 use as its DTE device?

  - telco incoming router

  - packet switcher

  - analog modem

  - PAD

- When you have multiple circuits connected to a site, the entire circuit is known as a ________.

  - virtual circuit

  - emulated circuit

  - joined circuit

  - multitasked circuit

- You have Port Security configured on your switch port to lock it to a single MAC address that you have statically configured.  You unplug your computer and plug in an IP phone.  What happens by default?

  - The switch port blocks the phone but would permit the computer

  - The switch port is shut down

  - The switch port permits the traffic, but alerts you

  - The switch port redirects the traffic

- Refer to the exhibit in the box below.  What is the problem with the access list shown?

  - The access list is permitting the host traffic before denying it

  - The access list cannot have more than two entries

  - The access list must end with a deny statement

  - The access list is fine

- How many B channels does an ISDN PRI support?

  - 2

  - 8

  - 23

  - 48

- What port does L2TP use?

  - 501

  - 1723

  - 1701

  - 443

- Refer to the exhibit in the box below.  What command produced the following results?

  - nbtstat

  - traceroute

  - pathping

  - netstat

- What type of firewall works on the Session layer that creates a connection and allows packets to flow between two hosts without further checking?

  - proxy server

  - application firewall

  - NAT filtering

  - circuit-level gateway

- When trying to protect your network, you should create your secure network based on ___________.

  - multiple content filters

  - a master firewall that all traffic flows through

  - a NAT firewall and a proxy server

  - layers

- You use a cable tester to discover that pin 1 connects to pin 2 in your cable.  What is this called?

  - Open

  - Attenuation

  - Crosstalk

  - Short

- What is a rogue access point?

  - An access point that is not authorized by your organization

  - An access point that is misconfigured

  - An access point for another SSID

  - An access point for another channel

- Which of the following should you protect against if you are adding a new AP to extend the reach of your SSID?

  - Reconnaissance system

  - Man-in-the-middle system

  - Botnets

  - Worms

- Which IPSec component provides connectionless integrity and data authentication but does not provide confidentiality?

  - SA

  - AH

  - ESP

  - MPPE

- What port does DNS use?

  - 443

  - 389

  - 51

  - 53

- The MAC address is associated with which layer of the OSI model?

  - Layer 1

  - Layer 2

  - Layer 3

  - Layer 4

- What transmission method does 802.11b use?

  - MU-MIMO

  - MIMO

  - OFDM

  - DSSS

- What feature within Windows allows the server to act as a router?

  - IPSec

  - DHCP

  - IP forwarding

  - RDC

- Which IPSec component includes the most security, including confidentiality?

  - SA

  - AH

  - ESP

  - MPPE

- What frequency band does 802.11a use?

  - 2.4 GHz

  - 5 GHz

  - 10 GHz

  - 4 GHz

- What command can you use to connect to a mail server so that you can test SMTP?

  - ftp

  - nslookup

  - telnet

  - nbtstat

- What command do you use to display and modify the network configuration of a local computer?

  - netsh

  - netstat

  - telnet

  - nbtstat

- What servicing of your data center environment might involve chemicals, gas, and electricity?

  - Fire suppression

  - HVAC

  - Power

  - Load

- What command is used to add static routes to a Windows computer?

  - nslookup

  - telnet

  - route

  - nbtstat

- What protocol do you use to remote to a Windows desktop from another location?

  - SIP

  - RTP

  - RDP

  - SNMP

- Which is a DNS record for an e-mail server?

  - CNAME

  - SOA

  - NS

  - MX