Contestant Number: ____________________

Time: ___________________

Rank: _______________

Contestant Number: ____________________

Time: ___________________

Rank: _______________

COMPUTER NETWORK TECHNOLOGY

(300)

NATIONAL 2021

TOTAL POINTS				__________ (100 points)

Test Time:  60 minutes

Choose the best answer to the following questions. Mark A if the statement is true. Mark B if the statement is false.

- When making a request of the OS, an API calls a method.

  - True

  - False

- When transmissions are missing packets at the destination, what layer of the OSI handles how the data is recovered and reassembled?

  - Application layer

  - Presentation layer

  - Session layer

  - Transport layer

- The address fe80::8cf1:2c42:ffde:da1c is a multicast address.

  - True

  - False

- Describe the physical appearance of a bus topology network.

  - Devices are connected to two adjacent devices, and communication priority is granted by a token

  - Devices are connected directly to a centralized networking device, known as a network switch

  - Devices are daisy-chained together in a single line

  - Devices are directly attached to a network router, which forwards data to intended destinations

- What type of DNS record will hold SPF and DKIM record information?

  - SRV record

  - CNAME record

  - TXT record

  - PTR record

- In a forward DNS zone, what is the function of the MX record?

  - It identifies the hostname and port of a computer that hosts a specific network service, such as FTP or SIP

  - It identifies the authoritative name server for a domain

  - It identifies the e-mail server to be used for e-mail traffic for the domain

  - It holds the alternative names for a host

- What can be done to minimize crosstalk in twisted pair cabling?

  - No more than 1 inch of the cable should be exposed

  - No less than 1 inch of the cable should be exposed

  - Each pair should be stripped of insulation so that it doesn’t get caught in the jack

  - Each pair should be twisted around another pair to reduce cross talk

- What dig command allows you to test the resolution of your domain thiscompany.com MX record using Google’s DNS server with the IP address of 8.8.8.8?

  - dig –server 8.8.8.8 –type=mx thiscompany.com

  - dig –s 8.8.8.8 –t mx thiscompany.com

  - dig query 8.8.8.8 domain thiscompany.com type mx

  - dig @8.8.8.8 thiscompany.com MX

- What does a jumbo frame enabled network mean?

  - The MTU for the network can be as high as 9198 bytes

  - The network is not based on the Ethernet standard

  - Fragmented frames will be consolidated into whole frames before being sent

  - The MTU for the network is set at 65,535 bytes

- Electrical components can be damaged in as little as what voltage?

  - 5 volts

  - 10 volts

  - 100 volts

  - 1500 volts

- What light wavelength encompasses 1000Base-LX?

  - 850 nanometers

  - 1200 nanometers

  - 1300 nanometers

  - 1550 nanometers

- What is the correct pinout order for T568B, beginning at pin 1?

  - white/green, green, white/orange, blue, white/blue, orange, white/brown, brown

  - blue, white/blue, green, white/green, orange, white/orange, brown, white/brown

  - white/orange, orange, white/green, blue, white/blue, green, white/brown, brown

  - white/brown, brown, white/green, blue, white/blue, green, orange, white/orange

- What technology separates bidirectional data in Bidirectional transceivers?

  - wavelength division multiplexing

  - time division multiplexing

  - frequency division multiplexing

  - statistical time division multiplexing

- Which protocol sends a signal during initial connection but does not transfer data during the session?

  - NTP

  - LDAP

  - TFTP

  - SIP

- Which service allows users to access their resources on any computer in the domain by simply signing in with their network credentials?

  - Active Directory Federated Users (AD FU)

  - Active Directory Domain Services (AD DS)

  - Automated Directory Network Services (AD NS)

  - Windows Named Resource Services (WN RS)

- Describe the function of the Layer 2 LLC sublayer.

  - It is used to handle multiplexing, flow and error control, and reliability

  - It is used to manage MAC addresses in message frames

  - It performs management of the physical layer’s modulation techniques

  - It is used to manage low-level encryption on a network

- Cisco switches do not include ______ as a pre-established VLAN.

  - VLAN 1

  - VLAN 1001

  - VLAN 1003

  - VLAN 1005

- Where is the DMARC located in a structured cabling environment?

  - in the Main Distribution Frame (MDF)

  - at the Intermediate Distribution Frame (IDF)

  - in the work area

  - between the MDF and the IDF

- An STP, non-root bridge needs to forward traffic to the root bridge. Which port can accomplish this task?

  - Only one root port, which is the bridge’s port that is closest to the root bridge, can forward

  - Only one root port, which is the bridge’s port that is furthest from the root bridge, can forward

  - All ports can forward frames to the root bridge, provided they are not in a down state

  - All ports will forward frames to the root bridge, unless a BPDU is received back on that same port

- Why would you use root guard in configuring a switch?

  - You wish to block BPDUs on an access port serving network hosts

  - You wish to disable STP on a port connected to a partnered company’s switch

  - You wish to prevent switches beyond a certain port from becoming the root bridge, but still wish to use STP

  - You wish to prevent a rogue switch or computer from hijacking the network’s STP paths

- Uses the 6-bit Differentiated Services field in the IP header for the purpose of packet classification.

  - Priority Code Point (PCP)

  - Differentiated Services Code Point (DSCP)

  - Class of Service (CoS)

  - Forward Error Correction (FEC)

- Describe the purpose of the Presentation layer in the OSI model.

  - The Presentation layer describes the interface between two applications, each on separate computers

  - The Presentation layer is responsible for reformatting, compressing, and/or encrypting data in a way that the application on the receiving end can read

  - The Presentation layer is responsible for describing how data between applications is synced and recovered if messages don’t arrive intact at the receiving application

  - The Presentation layer is responsible for transporting Application layer payloads from one application to another

- Mail messages are sent to a server using this protocol.

  - POP3

  - IMAP4

  - SMTP

  - HTTPS

- What is considered the most popular web server application?

  - Microsoft Internet Information Services

  - NGINX

  - Lighttpd

  - Apache

- The TCP/IP model often ignores this layer because it is considered simple.

  - Application layer

  - Network layer

  - Physical layer

  - Data link layer

- What federal agency is charged with safety and health in the workplace?

  - Occupational Safety and Health Administration (OSHA)

  - Workplace Safety and Hazard Administration (WSHA)

  - Office Safety and Standards Department (OSSD)

  - Hazardous Materials and Safety Management (HMSM)

- Errors and shorted component life, caused by static discharge, leads to this type of failure.

  - catastrophic failure

  - interrupting failure

  - upset failure

  - temporary failure

- There are two types of fiber-optic cable, single-mode fiber (SMF) and multimode fiber (MMF).

  - True

  - False

- What describes backbone cabling the best?

  - Short length cables with connectors at both ends

  - It is cabling that connects workstations to the closest data room and to switches housed in the room

  - The cables or wireless links that provide interconnection between the entrance facility and MDF, and between the MDF and IDFs

  - The shielded cables that are used for high data transmission rates between an organization and an ISP

- Which part is the domain name in ftp1.omaha.thiscompany.com?

  - ftp1

  - ftp1.omaha

  - ftp1.omaha.thiscompany

  - thiscompany.com

- IPv6 addresses are represented as nine blocks of hexadecimal numbers separated by colons.

  - True

  - False

- The extension identifier is found in ______.

  - The first 24 bits of the MAC address

  - The last 24 bits of the MAC address

  - The first 12 bits of the MAC address

  - The last 12 bits of the MAC address

- What is the IP range for a Class B?

  - 1.x.y.z to 126.x.y.z

  - 128.0.x.y to 191.255.x.y

  - 192.0.0.x to 223.255.255.x

  - 224.x.y.z to 255.x.y.z

- The NIC’s MAC address is used to assign a unique 64-bit address in this standard.

  - EUI-64

  - IEEE 802.36

  - UUID-128

  - MACin6

- Which field advises routers of incoming IPv4 packet priority?

  - Differentiated Services (DiffServ)

  - Internet header length (IHL)

  - Time to Live (TTL)

  - Padding

- Which additional protocol is required to guarantee the delivery of data, other than the IP connectionless protocol?

  - UDP

  - ICMP

  - TCP

  - ARP

- What is the result when a network collision occurs?

  - The collision goes undetected, and data transmission continues

  - The collision will create an error in the network switch, but otherwise, no issues will occur as a result

  - Each node on the network stops transmitting, until manually told to reconnect and transmit

  - Each node on the network waits a random amount of time and then resends the transmission

- Gigabit speeds are supported on ______ UTP and above.

  - Cat 3

  - Cat 5

  - Cat 5e

  - Cat 6

- What is the RG rating in coaxial cabling?

  - The number of twists in the copper core

  - The overall thickness of the cable

  - The maximum distance the cable is rated for, in kilometers

  - The materials used for shielding and conducting cores

- What is “jitter”?

  - Jitter is what happens when packets experience varying amounts of delay and arrive out of order

  - Jitter is the loss of 50% or more packets in a given data stream, causing a connection to falter

  - Jitter is the fluctuation of a signal’s strength as it travels away from its source

  - Jitter is the transmission of signal onto unintended pairs in a copper cable

- Which frame type assists in the delivery of data frames and provides MAC-layer reliability functions?

  - management frames

  - control frames

  - data frames

  - extended frames

- What field indicates a size of 8 to 10 bytes in PPP headers and trailer of the Network Layer packets?

  - priority

  - FCS

  - FEC

  - encryption

- What is the dotted decimal form for a CIDR notation of /26?

  - 255.255.255.0

  - 255.255.255.192

  - 255.255.255.224

  - 255.255.255.240

- VLAN hopping attacks can be circumvented by ______.

  - Use MAC address filtering

  - Install a router to process the untagged traffic on the VLAN

  - Install an additional switch to isolate traffic

  - Disable auto trunking and move native VLANs to unused VLANs

- Find the network ID of the host with an IP address of 172.16.1.144 /23.

  - 172.16.0.0

  - 172.16.1.0

  - 172.16.2.0

  - 172.0.0.0

- Class B networks contain this many max hosts IP addresses. What is the maximum number of host IP addresses that can exist in a Class B network?

  - 1022

  - 8190

  - 32,766

  - 65,534

- Which is enabled to block BPDUs, preventing ports as consideration for best path?

  - BPDU filter

  - BPDU guard

  - root guard

  - BPDU drop

- Which can find and fix link failures and is a descendant of STP?

  - Transparent Interconnection of Lots of Links (TRILL)

  - Shortest Path Bridging (SPB)

  - Rapid Spanning Tree Protocol (RSTP)

  - Multiple Spanning Tree Protocol (MSTP)

- Which is a significant downfall to WEP?

  - It did not allow the use of a password for access to the network

  - It provided no encryption for traffic sent over the air

  - It used a shared encryption key for all clients, and the key might never change

  - It only encrypted the initial connection authentication, but did not encrypt subsequent traffic

- Regarding DoS attacks, which statement is correct?

  - A denial-of-service attack occurs when a MAC address is impersonated on the network

  - A denial-of-service attack prevents legitimate users from accessing normal network resources

  - A denial-of-service attack is generally a result of a disgruntled employee

  - A denial-of-service attack is no longer a major concern due to the increased throughput available on most networks