Contestant Number: ____________________

Time: ___________________

Rank: _______________

Contestant Number: ____________________

Time: ___________________

Rank: _______________

COMPUTER NETWORK TECHNOLOGY

(300)

STATE 2021

TOTAL POINTS				__________ (100 points)

Test Time:  60 minutes

Choose the best answer to the following questions. Mark A if the statement is true. Mark B if the statement is false.

- A PDU is also known by this name when referencing the Data Link layer.

  - packet

  - data

  - bit

  - frame

- All of the following are disadvantages of peer-to-peer networks except they______.

  - lack scalability

  - are not necessarily secure

  - are impractical for connecting large numbers of computers

  - centralize user account logins

- Concerning twisted-pair cabling, at what maximum bend radius will data transmission most likely be disrupted?

  - No more than two times the diameter of the cable

  - No more than four times the diameter of the cable

  - No more than ten times the diameter of the cable

  - Twisted pair cable has a near infinite bend radius

- ______is also known as a graphical representation of the infrastructure to a wired network.

  - network diagram

  - wiring schematic

  - topology map

  - system diagram

- In VoIP, which device converts signals from analog phone equipment to IP data to travel over telephone lines?

  - VoIP PBX

  - VoIP MTA

  - VoIP gateway

  - VoIP translator

- Which of the following is used to communicate with applications running on hosts at the Transport layer of the OSI model?

  - IP addresses

  - port numbers

  - domain names

  - MAC addresses

- The command to display the configuration of TCP/IP for the network adapters installed.

  - ipconfig/show

  - ipconfig/list

  - ipconfig/all

  - ipconfig/full

- The IPv6 address representing a global unicast address is ______.

  - FE80::10

  - 2000::/3

  - FC00::/7

  - FD00::/8

- The tracert utility in Windows and the traceroute utility in Linux have similarities and differences. Which statement is true concerning one of their differences?

  - Only tracert can sent UDP messages for tracing a path.

  - By default, the tracert utility uses ICMP echo requests, while traceroute uses UDP datagrams or TCP SYN messages.

  - The Windows tracert utility does not place limits on the TTL of repeated trial messages.

  - The tracert utilty expects an ICMP port unreachable error message as the final reply to a trace.

- IP does not establish a session to send packets.

  - True

  - False

- What is the result of a router receiving a packet with a TTL equal to 0?

  - The router drops the packet and sends an ICMP TTL expired message back to the host.

  - The router attempts to forward the traffic on a local network.

  - The router resets the TTL to 128.

  - The router marks the packet as corrupted and forwards it to the next hop.

- Which unit is employed to measure the distortion of a signal on a network?

  - joules

  - volts

  - decibels

  - farads

- Which twisted-pair cabling standard must be utilized for wiring a new government building?

  - TIA/EIA 568A

  - TIA/EIA 568B

  - TIA/EIA 568F

  - TIA/EIA rollover

- Which of the following Ethernet cabling standard employs the fastest speed?

  - 100Base-T

  - 1000Base-T

  - 10GBase-T

  - 10TBase-T

- When examining 802.11, at which layer of the OSI model might you see the standards vary.

  - Physical Layer

  - Network Layer

  - Data link Layer

  - Transport Layer

- When using a Linux device to connect to an SSH server, what location must house the public key before the device is allowed to connect?

  - In an authorization file under your home directory on your computer

  - In an authorization file on the host where the SSH server is

  - In the /etc/ssh/keys folder

  - In the /var/run/ssh/public folder

- The IPsec protocol operates at this layer of the OSI model.

  - Physical Layer

  - Network Layer

  - Transport Layer

  - Application Layer

- Two different, native VLAN connected access ports that both tag traffic with different VLAN IDs will trigger a VLAN mismatch.

  - True

  - False

- The CIDR notation for a subnet of 255.255.248.0.

  - /18

  - /20

  - /21

  - /29

- Networks are typically segmented in all of the following ways except by ______.

  - geographic location

  - departmental boundaries

  - device types

  - device manufacturer

- The words malicious and software combine to form the term malware.

  - True

  - False

- Which definition explains the function of a worm?

  - A program that disguises itself as something useful but actually harms your system.

  - A process that runs automatically, without requiring a person to start or stop it.

  - A program that runs independently of other software and travels between computers and across networks.

  - A program that locks a user’s data or computer system until a ransom is paid.

- By definition, which is considered a phishing attempt?

  - An employee at your company has received a malware-infected file in their e-mail.

  - A person posing as an employee tried to access a secured area at your organization.

  - A gift was offered to an employee with access to secured information in exchange for details.

  - An e-mail was sent to a manager at your company that appeared to be from the company’s CTO asking for access.

- What is the equivalent wildcard mask value of using the “any” keyword when used in access control lists?

  - 0.0.0.0

  - 255.255.255.255

  - 255.255.0.0

  - 0.0.255.255

- What is the first step in STP path selection?

  - STP must first select the root bridge, or master bridge.

  - STP examines the possible paths between all other bridges.

  - STP disables links that are not part of a shortest path.

  - STP begins to block BPDUs on non-designated ports.

- A leased frame relay circuit holds this advantage.

  - You are guaranteed to receive the maximum amount of bandwidth specified in the circuit contract.

  - You pay only for the bandwidth you require.

  - The paths that your data will take are always known.

  - Frame relay is a newly established network technology with more features than other technology.

- After identifying the problem, a theory to a probable cause of a problem should be established.

  - True

  - False

- What type of failure is caused by an open electrical circuit in a failed circuit breaker?

  - fail-open

  - fail-close

  - fail-tolerant

  - fail-dynamic

- What is the Presentation layer responsible for in the OSI model?

  - The Presentation layer describes the interface between two applications, each on separate computers.

  - The Presentation layer is responsible for reformatting, compressing, and/or encrypting data in a way that the application on the receiving end can read.

  - The Presentation layer is responsible for describing how data between application is synced and recovered if messages don’t arrive intact at the receiving application.

  - The Presentation layer is responsible for transporting Application layer payloads from one application to another.

- Which of the following is an example of encapsulation?

  - The addition of a header to data inherited from the layer above in the OSI model

  - The subtraction of a header from data inherited from the layer below in the OSI model

  - The modification of headers from a higher layer in the OSI model

  - The addition of a trailer to data inherited from the layer above in the OSI model

- Which layer of the OSI model do the TCP and UDP protocols operate?

  - Application Layer

  - Presentation Layer

  - Transport Layer

  - Network Layer

- In a rack, multiple consoles can connect to a single device using a KVM.

  - True

  - False

- A cable bend radius can be best defined as a ______.

  - cable bend radius is the radius of the maximum arc into which you can loop the cable without impairing data

  - cable bend radius is the radius of the minimum arc into which you can loop the cable without impairing data

  - cable bend radius determines the total number of bends that can exist in an overall cable before data is impaired

  - cable bend radius defines the twist rate of the cable inside of the insulation

- In a network diagram, which symbol represents a network switch?

  - An icon that has a hockey-puck shape with two arrows pointing inward and two arrows pointing outward

  - An icon that resembles a brick wall

  - An icon that is rectangular in shape, with squiggly lines on the front

  - An icon that is rectangular, which contains four arrows pointing in opposite directions

- TLD servers can be located at the root level of the DNS hierarchical structure.

  - True

  - False

- The NIC of a computer has an IP of 169.254.196.200. What can the technician assume from this information?

  - The computer has been assigned a routed public IP address.

  - The network card has been erroneously assigned a loopback address.

  - The computer is configured to use DHCP, but was unable to lease an address.

  - The network card is set up for multicast communication.

- All IP address ranges are recommended for use private networks except ______.

  - 10.0.0.0 through 10.255.255.255

  - 172.16.0.0 through 172.31.255.255

  - 192.168.0.0 through 192.168.255.255

  - 127.0.0.0 through 127.255.255.255

- During DHCP transmission, clients receive responses on which port?

  - port 67

  - port 68

  - port 546

  - port 547

- BGP is accurately defined by the following.

  - BGP is limited to a single autonomous system.

  - BGP is exclusively a distance-vector protocol.

  - BGP utilizes TCP for communicating updates.

  - BGP is a more advanced version of OSPF.

- If duplicate MAC addresses exist on a switched network, what will the outcome be?

  - The hosts that share the same MAC addresses will be completely unable to communicate with any other devices.

  - The hosts will generate new MAC addresses until the conflict is resolved.

  - The hosts will still send and receive traffic, but traffic may not always reach the correct destination.

  - The network switch will eventually crash due to being unable to properly forward traffic.

- Flaws in transmission from attenuation is most likely caused by ______.

  - a customer modem is continuously losing signal due to large distance from the transmitting device

  - a network switch interface is dropping due to a powerful radio transmitter in the adjacent room

  - a customer’s video conferencing application is experiencing delay in audio and video that comes and goes

  - two network interfaces are experiencing problems at the same time due to the cables having adjacent runs

- How deep should the sheath of an Ethernet cable extend into the terminating RJ-45 connector?

  - 1/2 inch

  - 3/8 inch

  - 4/5 inch

  - 1/5 inch

- A shared access point, by a group of workstations, is called a(n) ______.

  - extended service set

  - basic service set

  - modified service set

  - generic service set

- What is the frequency range for the wireless spectrum, as defined by the FCC?

  - 9 kHz and 300 GHz

  - 125 kHz and 150 GHz

  - 3 kHz and 500 GHz

  - 3 MHz and 300 GHz

- The term used to define a service provider that rents space at a data center is ______.

  - point of presence (PoP)

  - service location (SL)

  - central service point (CSP)

  - locally exchanged data point (ledp)

- The amount of /64 subnets that can be created within /56 prefix is ______.

  - 256

  - 512

  - 1024

  - 2048

- Where is the VLAN database stored in VTP?

  - On the router responsible for maintaining the VTP database

  - On the switch that is configured as the trunk root

  - On the switch that is known as the stack master

  - On the designated VLAN server for the network

- Which legacy protocol requires both devices to authenticate?

  - Password Authentication Protocol (PAP)

  - Challenge Handshake Authentication Protocol (CHAP)

  - Microsoft Challenge Handshake Authentication Protocol (MS-CHAP)

  - Microsoft Challenge Handshake Authentication Protocol, version 2 
(MS-CHAPv2)

- Which of the following statements best describes role-based access control?

  - RBAC allows a network administrator to base privileges and permissions around a detailed description of a user’s roles or jobs.

  - RBAC allows users to decide for themselves who has access to that user’s resources.

  - RBAC organizes resources into hierarchical classifications, such as “confidential” or “top secret”.

  - RBAC is the most restrictive method of access control.

- Cable management is not important in a rack system.

  - True

  - False