Contestant Number: ____________________

Time: ___________________

Rank: _______________

NETWORK ADMINISTRATION USING CISCO

(315)

STATE 2021

TOTAL POINTS				__________ (100 points)

Test Time:  60 minutes

Please select the answer that best answers the question.

- What command is used to encrypt all passwords on the Cisco switch?

  - service password-encryption

  - enable secret level 15 0 <password>

  - encrypt password all

  - keychain encrypt all

- What command is used to clear all mac address table entries?

  - clear mac-address-table

  - erase mac-address-table

  - delete mac-address-table

  - remove mac-address-table

- What command enables an instance of OSPF on a Cisco router?

  - router ospf

  - ip routing

  - ip router opsf 1

  - ip route open-shortest-path-first

- What command is used to display the current status of Spanning-Tree and how it is affecting ports on the Cisco Switch?

  - show spanning-tree

  - show spanning-tree online

  - show port-forwarding

  - show pvlan-spanning-tree

- What command is used to configure the Cisco switch to use rapid per-vlan spanning-tree protocol?

  - enable rapid-pvst

  - set spanning-tree mode rapid-pvst

  - spanning-tree rapid-pvst enable

  - spanning-tree mode rapid-pvst

- What subnet mask could be applied to the network 192.168.10.0 which would most closely allow 29 hosts on the subnetwork?

  - 255.255.255.0

  - 255.255.255.192

  - 255.255.255.240

  - 255.255.255.224

- What subnet mask could be applied to the network 192.168.25.0 which would closely create at least 9 subnetworks?

  - 255.255.255.224

  - 255.255.255.192

  - 255.255.255.240

  - 255.255.255.128

- Which IP Addresses properly exists in the network 192.168.52.16/28?

  - 192.168.52.29

  - 192.168.52.32

  - 192.168.52.10

  - 192.168.52.33

- Which IP Address would be able to communicate with the IP Address 192.168.39.147/26?

  - 192.168.39.87

  - 192.168.39.129

  - 192.168.39.202

  - 192.168.39.1

- Which of the following would not be a network ID produced when using the IP network 192.168.4.0 and the CIDR mask /27?

  - 192.168.4.16

  - 192.168.4.32

  - 192.168.4.64

  - 192.168.4.128

- When connecting a host to a port on a switch, the port does not turn on. Which of the following is the most likely cause?

  - RIP has disabled the port.

  - Spanning-Tree has marked the port as a designated port.

  - The cable between the host and the switch is not a straight cable.

  - The port on the switch has not been enabled with the command “turn on port”.

- With a new host that is connected to a switch and given an appropriate IP Address, it is not able to reach an existing host that is on the same IP network. What is the most likely reason it cannot communicate?

  - The port on the switch that the new host is connected to is in another VLAN.

  - The default gateway for the new host is set incorrectly.

  - The existing host does not have a route back to the new host.

  - OSPF is blocking the port to the new host.

- When configuring a new trunk between two switches, hosts on all VLANs except for the Marketing VLAN are able to communicate with their other respective hosts. What is the most likely reason the Marketing VLAN cannot communicate between the two switches?

  - The “no shut” command has not been entered for the VLAN on both switches.

  - A firewall on the switch is preventing the VLAN from communicating.

  - The trunk port is not in access mode for the Marketing VLAN.

  - The Marketing VLAN was not added to one or both sides of the trunk connection.

- On a Cisco router, after setting the IP Address on an interface, you are not able to ping it. What is most likely the problem?

  - Spanning-tree has disabled the port.

  - The IP Address assigned to the port is a network ID.

  - The port has not been enabled with the “no shut” command.

  - RIP has not been enabled on the router.

- A Cisco router has its interface configured for routing which includes having an IP Address set on it and the port being enabled. However, an entry for the new network is not added into the routing table. What is the most likely issue?

  - The port is not physically connected.

  - The routing table is full and cannot accept new entries.

  - The port has an IP address on it that is in the same network as another port on the router.

  - The command “enable routing” has not been configured on the port.

- Which commands below configures a port on a Cisco switch to be a trunk port and allow just VLANs 10 and 20?

  - switchport trunk enable; switchport trunk enable vlan 10,20

  - switchport mode trunk; switchport trunk allow vlan 10,20

  - switchport mode trunk; switchport trunk allow all

  - switchport trunk enable; switchport trunk enable all

- In the sense of Spanning-Tree Protocol, what is the effect of enable “PortFast” on an interface?

  - It enables accelerated transfer speeds on the interface, thereby lowering the port cost.

  - It removes the interface from the Spanning-Tree environment.

  - It makes the interface skip the listening and learning states and go straight to the forwarding state.

  - It enables Rapid Spanning-Tree Protocol on the interface.

- On a Cisco switch, you run the command “show spanning-tree active” and receive the following output:

Interface 		Role Sts Cost 		Prio.Nbr Type

---------------- 	---- --- --------- 	-------- ----------Fa0/13 			Altn BLK 19 		128.13 	P2p

Fa0/11			Root FWD 19		128.11	P2p

What does the role “Root” most likely mean?

  - This port is used to connect this switch to the upstream switch in the direction of the root switch.

  - This port is used by downstream switches to connect to it since it is the root switch.

  - This port is the bottom of the spanning-tree environment.

  - This port is being blocked to prevent a layer-2 loop.

- On a Cisco switch, you run the command “show spanning-tree active” and receive the following output:

Interface 		Role Sts Cost 		Prio.Nbr Type

---------------- 	---- --- --------- 	-------- ---------- Fa0/13			Desg FWD 19		128.13   P2p

Fa0/16			Desg FWD 19		128.16   P2p

What does the role “Desg” most likely mean?

  - This port is used to connect this switch to the upstream switch in the direction of the root switch.

  - The port is being blocked to prevent a layer-2 loop.

  - Port 13 and 16 are connected to one another using a cross-over cable.

  - This port is used by downstream switches to connect to it since it is in the direction of the root switch.

- What are the states of 802.1D Spanning-Tree?

  - Disabled, Blocking, Listening, Learning, Forwarding

  - Discarding, Learning, Forwarding

  - Removing, Accepting, Learning, Forwarding

  - Disabled, Enabled

- Which of the following is the IEEE standard for Rapid Spanning-Tree?

  - 802.1w

  - 802.1d

  - 802.11

  - 802.11a

- On a Cisco switch, you run the command “show spanning-tree active” and receive the following output:

Switch#show span active

VLAN0001

Spanning tree enabled protocol ieee

Root ID 	Priority 32769

Address 0009.7CC4.EBE4

Cost 19

Port 11(FastEthernet0/11)

Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec

Bridge ID 	Priority 32769 (priority 32768 sys-id-ext 1)

Address 0010.1148.23B8

Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec

Aging Time 20

What is the significance of “Address 0009.7CC4.EBE4”?

  - It is the MAC address of this switch.

  - This is the MAC address of root switch in the spanning-tree environment.

  - It is the MAC address of the next downstream switch.

  - It is the MAC address of the host connected to port FastEthernet0/11.

- What are the states of the 802.1W Rapid Spanning-Tree?

  - Discarding, Learning, Forwarding

  - Disabled, Blocking, Listening, Learning, Forwarding

  - Removing, Accepting, Learning, Forwarding

  - Disabled, Enabled

- In the sense of Rapid Spanning-Tree, what does the port role “Alternate” mean?

  - This port is the backup “root” port, to be used if the current root port fails.

  - This port is the backup “designated” port, to be used if the current designated port fails.

  - This signifies that this switch will take over as root if the root switch fails.

  - This signifies that this switch will take over as the designated switch if the current designated switch fails.

- In the sense of Rapid Spanning-Tree, what does the port role “Backup” mean?

  - This port is the backup “root” port, to be used if the current root port fails.

  - This signifies that this switch will take over as root if the root switch fails.

  - This port is the backup “designated” port, to be used if the current designated port fails.

  - This signifies that this switch will take over as the designated switch if the current designated switch fails.

- Router Information Protocol (RIP) is a Distance Vector Routing Protocol.

  - True

  - False

- What is the maximum number of hops allowed in a routing environment using RIP?

  - 8

  - 15

  - 16

  - 32

- What is the correct command to enable RIP on an interface which has an IP Address of 192.168.37.1?

  - network 192.168.37.1 255.255.255.0

  - network 192.168.37.0 0.0.0.255

  - network 192.168.37.1/24

  - network 192.168.37.0

- When looking at the output of “show ip route”, what code indicates a route learned by RIP?

  - S

  - C

  - R

  - O

- Open Shortest Path First (OSPF) is a Distance Vector Routing Protocol.

  - True

  - False

- What information is shared within a single area of OSPF?

  - Link-states within the databases of all routers in the area

  - Network summaries of all networks for the routers in the area

  - Router management IP Addresses for routers in the area

  - Distance between the router interfaces

- What is the correct command to enable single area OSPF on an interface which has an IP Address of 192.168.37.1/24?

  - network 192.168.37.0 0.0.0.255 area 0

  - network 192.168.37.0 area 0

  - network 192.168.37.0 255.255.255.0 area 0

  - network 192.168.37.0 0.0.0.255

- When turning on OSPF for a router using the command “router ospf 1”, what does the “1” signify?

  - The Process ID for OSPF on this router.

  - The autonomous system number which should match all other routers in the area.

  - The priority for being the designated router.

  - This will be the first router with OSPF enabled.

- When a router has learned the same route using RIP and OSPF, OSPF will take precedence.

  - True

  - False

- With OSPF, if the Router-ID has not manually been set, what is the next source it will use for its Router-ID?

  - Highest IP Address assigned on the device

  - IP Address of Loopback Interface

  - Lowest IP Address assigned on the device

  - IP Address of Management Interface

- What range of values are used to create a standard access control list?

  - 100-199

  - 1-499

  - 1-99

  - 500-999

- Standard access control list can be used to identify traffic based on destination information.

  - True

  - False

- Which option is commonly used to identify a single IP Address in an access control list?

  - any

  - subnet mask

  - host

  - wildcard mask

- What is the effect of the following access control list entries (assuming they have the same ACL number)?

deny ip host 192.168.52.70

permit ip 192.168.52.0 0.0.0.255

  - It will block address 192.168.52.70 but allow all other traffic.

  - It will block all 192.168.52.0/24 traffic.

  - It will block all traffic except for 192.168.52.70.

  - It will allow all traffic.

- When placing an ACL on an interface, what direction is the ACL allowed to check?

  - In or out

  - In

  - Out

  - Away

- There is a requirement that all management communication made to the Cisco device must be secure. What protocol should be enabled and used?

  - Telnet

  - HTTP

  - FTP

  - SSH

- What port number is used by SSH?

  - 21

  - 22

  - 80

  - 443

- What port number is used by Telnet?

  - 21

  - 22

  - 23

  - 80

- What port is commonly used for secure HTTP traffic?

  - 22

  - 80

  - 443

  - 8080

- FTP send data between the client and server securely.

  - True

  - False

- What is an advantage of using fiber optic cables instead of copper?

  - Communication can occur much greater distances.

  - It’s a common media type used by PCs and laptops.

  - It is easier to assemble connectors on cable than on copper cables.

  - They are more durable than copper cables.

- Based on the following starting network and subnet mask, which of the following addresses is not assignable to a device?

192.168.13.0 255.255.255.224

  - 192.168.13.64

  - 192.168.13.54

  - 192.168.13.34

  - 192.168.13.24

- Which command enables IPv6 forwarding on a Cisco router?

  - ipv6 routing enable

  - ipv6 enable

  - no ipv6 disable

  - ipv6 unicast-routing

- What command can be used to identify what VLANs are allowed on a trunk port?

  - show vlan

  - show interface

  - show interface switchport

  - show interface vlan

- What is the keyboard shortcut to return to privilege user mode directly?

  - Ctrl+Z

  - Ctrl+C

  - Ctrl+B

  - Ctrl+X