Member ID: ____________________

Time: ___________________

Rank: _______________

Computer Network Technology

(300)

National 2024

CONCEPT KNOWLEDGE:

Multiple Choice (50 @ 2 points each)	______________ (100 points)

Test Time: 60 minutes

Multiple Choice Questions

Directions: Identify the letter of the choice that best completes the statement or answers the question.

- What is the broadcast address for the IP network 192.168.1.0/24?

  - 192.168.1.0

  - 192.168.1.255

  - 192.168.0.1

  - 192.168.1.254

- What is the maximum number of IP addresses that can be assigned to hosts on a network with a subnet mask of 255.255.0.0?

  - 254

  - 510

  - 1022

  - 65534

- What port is used by SSH?

  - 20

  - 21

  - 22

  - 23

- Which port does RDP use?

  - 25

  - 110

  - 3389

  - 389

- Which port does MS SQL Server use?

  - 25

  - 1433

  - 443

  - 80

- The User Datagram Protocol (UDP) operates at which layer of the OSI model?

  - Layer 2 Data Link

  - Layer 3 Network

  - Layer 4 Transport

  - Layer 5 Session

- Which OSI model layer is responsible for establishing, managing, and terminating sessions between applications?

  - Layer 4 Transport

  - Layer 5 Session

  - Layer 6 Presentation

  - Layer 7 Application

- HTTP operates at which layer of the OSI model?

  - Layer 3 Network

  - Layer 4 Transport

  - Layer 5 Session

  - Layer 7 Application

- In which layer of the OSI model does the SSL/TLS protocol operate?

  - Layer 4 Transport

  - Layer 5 Session

  - Layer 6 Presentation

  - Layer 7 Application

- Which command is used to clear the DNS cache on a Windows system?

  - dnsflush

  - ipconfig /flushdns

  - flushdns

  - dnsreset

- What command is used to display the ARP (Address Resolution Protocol) cache on a Windows system?

  - arp

  - ifconfig /arp

  - netstat /arp

  - arp cache

- Which IEEE Wi-Fi standard is also known as Wi-Fi 6?

  - 802.11n

  - 802.11ac

  - 802.11ax

  - 802.11g

- Which Wi-Fi protocol introduced MIMO (Multiple Input Multiple Output) technology?

  - 802.11a

  - 802.11g

  - 802.11n

  - 802.11b

- Which one of the following frequency bands is not utilized by the Wi-Fi 6E (802.11ax) standard?

  - 2.4 GHz

  - 5 GHz

  - 6 GHz

  - 60 GHz

- In which network topology does the failure of a network device not affect the rest of the network?

  - Bus

  - Star

  - Mesh

  - Ring

- Which of the following IP address pairs are in the same subnet?

  - 192.168.0.1/24 and 192.168.0.100/24

  - 10.0.0.10/8 and 172.16.0.10/16

  - 192.168.1.1/24 and 192.168.2.1/24

  - 10.0.0.10/8 and 10.0.0.20/16

- Which of the following is a valid host IP in the 172.16.0.0/16 subnet?

  - 172.16.0.0

  - 172.16.255.255

  - 172.17.0.0

  - 172.16.255.0

- What is the subnet address for the IP 192.168.1.75/26?

  - 192.168.1.32

  - 192.168.1.64

  - 192.168.1.0

  - 192.168.1.128

- Which of these addresses is a private IP address?

  - 172.17.255.1

  - 172.32.255.1

  - 192.169.1.1

  - 200.200.200.200

- Which of the following is a Class B Private IP address?

  - 10.255.255.255

  - 172.16.0.0

  - 192.168.0.0

  - 169.254.0.0

- What does the /24 represent in the IP address 192.168.1.0/24?

  - Subnet mask

  - Number of subnets

  - Number of hosts

  - Network address

- What does a Proxy server do?

  - Encrypts data sent over a network

  - Serves as an intermediary for requests from clients seeking resources from other servers

  - Creates a virtual version of a physical network

  - Connects a user to the fastest available server

- What is the main difference between a VPN and a proxy server?

  - A VPN encrypts all of a device's internet traffic, while a proxy only affects specific applications or browsers

  - A proxy server encrypts all of a device's internet traffic, while a VPN only affects specific applications or browsers

  - A VPN and a proxy server do the same thing

  - There is no difference

- What is the primary function of the Domain Name System (DNS)?

  - Translating domain names to IP addresses

  - Increasing internet download speed

  - Providing email services

  - Encrypting data for secure transfer

- DNS servers that store the definitive versions of all DNS records for a domain are referred to as what?

  - Primary DNS servers

  - Authoritative DNS servers

  - Secondary DNS servers

  - DNS cache servers

- What do you call the process of updating all DNS servers across the internet with the latest DNS record information?

  - DNS Updating

  - DNS Spreading

  - DNS Propagation

  - DNS Broadcasting

- What protocol does DNS use for querying and updating records?

- HTTP and sometimes HTTPS

- UDP and sometimes TCP

- FTP and sometimes SFTP

- SMTP

- Which DNS record type is used to verify domain ownership?

- A

- CNAME

- TXT

- MX

- What is the function of an AAAA record?

- Map a hostname to an IPv6 address

- Map a hostname to an IPv4 address

- Verify domain ownership

- Point to a mail server

30.What DNS record type is used to define an alias for a domain name?

- NS record

- MX record

- CNAME record

- TXT record

31.What DNS record type is typically used for load balancing?

- CNAME record

- A record

- MX record

- SRV record

32.Which of the following is a key difference between DHCP and WINS?

  - DHCP is used for name resolution, while WINS is used for IP address assignment.

  - DHCP is primarily used in Windows networks, while WINS is used in Linux networks.

  - DHCP is a protocol, while WINS is a service provided by Microsoft.

  - DHCP operates at the application layer, while WINS operates at the transport layer.

33.WINS is most commonly used in which type of network?

- Peer-to-peer networks

- Virtual private networks (VPNs)

- Wide area networks (WANs)

- Local area networks (LANs)

34.When a client device receives an IP address from a DHCP server, how is it allocated?

- The client chooses an IP address from a predefined range.

- The DHCP server randomly selects an available IP address.

- The client device sends a request to the DHCP server, which assigns an IP address.

- The DHCP server assigns the next available IP address from a predefined range.

35.Which tool is commonly used to diagnose network issues by identifying the route taken by packets from the source to the destination?

- Wireshark

- Ping

- Traceroute

- Nmap

36.Which remote access tool is primarily used for remote desktop access and control?

  - VNC (Virtual Network Computing)

  - RDP (Remote Desktop Protocol)

  - Telnet

  - ICMP (Internet Control Message Protocol)

37.Which remote access tool’s current version is used for securely accessing email messages from a remote server??

- SMTP (Simple Mail Transfer Protocol)

- POP3 (Post Office Protocol 3)

- IMAP (Internet Message Access Protocol)

- FTP (File Transfer Protocol)

38.Which type of network operating system allows all devices on the network to have equal responsibilities and privileges?

  - Peer-to-peer

  - Client-server

  - Standalone

  - Mainframe

39.Which type of network operating system allows all devices on the network to have equal responsibilities and privileges?

- Peer-to-peer

- Client-server

- Standalone

- Mainframe

40.Which directory service is commonly used in Unix-like operating systems?

  - NIS

  - Novell eDirectory

  - OpenLDAP

  - Oracle Directory Server

41.Which directory service protocol uses port 389 by default?

- DNS

- RADIUS

- DHCP

- LDAP

42.Which of the following is true about IPv6?

  - It uses a 32-bit address format.

  - It uses a 128-bit address format.

  - It provides backward compatibility with IPv4.

  - It has a smaller address space than IPv4.

- What is the purpose of NAT (Network Address Translation)?

    - It provides security by hiding internal IP addresses.

    - It expands the address space of IPv6.

    - It enables automatic address configuration in IPv4.

    - It improves the efficiency of routing in IPv6.

- Which statement best describes an application-level firewall?

    - It filters traffic based on the IP addresses and port numbers of network packets.

    - It inspects the contents of network packets and makes decisions based on application-layer protocols.

    - It operates as an intermediary between internal clients and external servers.

    - It provides encryption and decryption services for secure data transmission.

- Which firewall technology allows or blocks traffic based on a set of predetermined rules?

    - Intrusion Detection System (IDS).

    - Intrusion Prevention System (IPS).

    - Unified Threat Management (UTM).

    - Access Control List (ACL).

- What is the purpose of a demilitarized zone (DMZ) in relation to firewalls?

    - It isolates internal networks from external networks.

    - It provides a secure area for testing firewall configurations.

    - It allows public-facing servers to be placed outside the internal network behind a firewall.

    - It acts as a backup location for firewall configurations.

- What is a potential drawback of using a host-based firewall?

- It can introduce network latency.

- It requires specialized hardware for deployment.

- It lacks centralized management capabilities.

- It only protects against external threats.

- What command is used to display the network configuration on a Linux system?

  - configip

  - ipconfig

  - netstat

  - ifconfig

- What command is used to display the ARP (Address Resolution Protocol) cache on a Windows system?

- arp

- ifconfig /arp

- netstat /arp

- arp cache

- Which of the following cloud solutions provides Infrastructure as a Service (IaaS)?

- Amazon Web Services (AWS)

- Salesforce

- Microsoft Office 365

- Dropbox