Member ID: ____________________

Time: ___________________

Rank: _______________

Computer Network Technology

(300)

State 2024

CONCEPT KNOWLEDGE:

Multiple Choice (50 @ 2 points each)	______________ (100 points)

Test Time: 60 minutes

Multiple Choice Questions

Directions: Identify the letter of the choice that best completes the statement or answers the question.

- What is the maximum number of IP addresses that can be assigned to hosts on a network with a subnet mask of 255.255.255.0?

A. 254

B. 510

C. 1022

D. 2046

- Which subnet mask would you use to create 32 subnets for a Class C network?

A. 255.255.255.192

B. 255.255.255.224

C. 255.255.255.240

D. 255.255.255.248

- Which of the following IP addresses is a local host address?

A. 192.168.1.1

B. 10.0.0.1

C. 127.0.0.1

D. 172.16.0.1

- Which port is used by HTTPS?

A. 23

B. 22

C. 80

D. 443

- What port does Telnet use?

A. 21

B. 22

C. 23

D. 25

- Which of the following is a function of the OSI model's Session layer?

A. IP addressing

B. Flow control

C. Dialogue control

D. MAC addressing

- At which layer of the OSI model would a network switch typically operate?

A. Layer 1 Physical

B. Layer 2 Data Link

C. Layer 3 Network

D. Layer 4 Transport

- Which OSI layer is responsible for data encryption, compression, and translation services?

A. Layer 3 Network

B. Layer 4 Transport

C. Layer 6 Presentation

D. Layer 7 Application

- Which OSI layer is responsible for end-to-end communication and error-free delivery of data?

A. Layer 2 Data Link

B. Layer 3 Network

C. Layer 4 Transport

D. Layer 5 Session

- What command is used to display active TCP or UDP connections?

A. nbtstat

B. netstat

C. ipconfig

D. nslookup

- What command is used to display the routing table on a Linux system?

A. route

B. ifconfig

C. ipconfig

D. netstat

- Which command is used to display the DHCP lease information on a Windows system?

A. dhcpcmd

B. ipconfig /renew

C. dhcpinfo

D. ipconfig /all

- Which command is used to display the network connection status on a Unix-based system?

A. netstat

B. ifstatus

C. statnet

D. connstatus

- In which frequency band does the 802.11b Wi-Fi protocol operate?

A. 2.4 GHz

B. 5 GHz

C. 6 GHz

D. 60 GHz

- Which IEEE Wi-Fi standard is also known as Wi-Fi 4?

A. 802.11g

B. 802.11n

C. 802.11ac

D. 802.11ax

- In which type of network topology does each device have exactly two neighboring devices?

A. Mesh

B. Bus

C. Ring

D. Star

- What would be the correct subnet mask for a network that requires 500 hosts?

A. 255.255.255.0

B. 255.255.254.0

C. 255.255.252.0

D. 255.255.248.0

- Which IP address does not belong to the subnet 192.168.1.0/24?

A. 192.168.1.255

B. 192.168.1.1

C. 192.168.2.1

D. 192.168.1.100

- What is the first valid host on the subnetwork that the node 172.18.40.56/22 belongs to?

A. 172.18.40.1

B. 172.18.40.0

C. 172.18.38.1

D. 172.18.39.1

- What is the correct broadcast address for the subnet 192.168.1.64/26?

A. 192.168.1.63

B. 192.168.1.95

C. 192.168.1.127

D. 192.168.1.255

- Which of the following is a Class A IP address?

A. 10.255.255.1

B. 172.16.0.1

C. 192.168.0.1

D. 169.254.0.1

- Which of these addresses can be used as a valid host IP in the subnet 192.168.1.0/25?

A. 192.168.1.0

B. 192.168.1.128

C. 192.168.1.126

D. 192.168.1.255

- Which of the following pairs of IP addresses are in the same subnet?

A. 192.168.1.10/24 and 192.168.1.100/24

B. 10.0.0.10/8 and 10.255.0.10/16

C. 172.17.0.1/16 and 172.16.0.1/16

D. 10.0.0.1/24 and 10.0.1.1/24

- Which of the following is the last IP address of the subnet 172.16.0.0/16?

A. 172.16.0.254

B. 172.16.255.254

C. 172.16.0.1

D. 172.16.255.1

- Which of the following best describes a VLAN (Virtual Local Area Network)?

A. A group of devices on separate physical networks that function as if they are on the   same network

B. A group of devices sharing the same IP address

C. A group of devices that share the same server

D. A group of devices that have the same operating system

- Which of the following protocols is commonly used by VPNs?

A. Hypertext Transfer Protocol (HTTP)

B. File Transfer Protocol (FTP)

C. Point-to-Point Tunneling Protocol (PPTP)

D. Simple Mail Transfer Protocol (SMTP)

- How does a VPN maintain user privacy?

- By blocking all internet traffic

- By changing the user's IP address

- By deleting the user's browser history

- By installing antivirus software on the user's device

- What is the primary function of a switch in a network?

- Distribute IP addresses

- Connect multiple networks

- Connect multiple devices on the same network

- Convert digital signals to analog

- Which device operates primarily at the data link layer (Layer 2) of the OSI model?

- Switch

- Router

- Modem

- Repeater

- Which of the following home devices would typically have a built-in DHCP server?

- Router

- Switch

- Modem

- Network card

- In a network, what is the main difference between a hub and a switch?

  - A hub operates on Layer 3 of the OSI model, while a switch operates on Layer 2

  - A hub broadcasts incoming traffic to all ports, while a switch only sends traffic to the intended recipient

  - A hub can route packets between different networks, while a switch cannot

  - A hub can provide wireless connectivity, while a switch cannot

- What is the DNS record type used to specify the mail server responsible for accepting email messages on behalf of a domain?

A. MX Record

B. SRV Record

C. TXT Record

D. A Record

- Which DNS record is used to delegate a DNS Zone to use the given authoritative name servers?

- NS

- A

- TXT

- CNAME

- What type of DNS record is used for reverse DNS (rDNS) lookups?

A. PTR record

B. MX record

C. A record

D. TXT record

- Which of the following is true about DHCP leases?

- Leases are permanent and never expire.

- Leases are manually assigned by network administrators.

- Leases determine the duration for which an IP address is assigned to a device.

- Leases are only used in wireless networks.

- WINS (Windows Internet Name Service) is used for:

- Assigning IP addresses to devices on a network.

- Resolving NetBIOS names to IP addresses.

- Configuring wireless network settings.

- Encrypting network traffic for enhanced security.

- When a client device receives an IP address from a DHCP server, how is it allocated?

- The client chooses an IP address from a predefined range.

- The DHCP server randomly selects an available IP address.

- The client device sends a request to the DHCP server, which assigns an IP address.

- The DHCP server assigns the next available IP address from a predefined range.

- Which tool is used to test network connectivity by sending echo requests and receiving echo replies?

- Wireshark

- Ping

- Traceroute

- Nmap

- Which tool is used to monitor network devices, collect performance data, and generate alerts?

- Ping

- SNMP

- ipconfig

- NetStumbler

- Which of the following remote access tools provides secure encrypted connections over the internet?

- VPN (Virtual Private Network)

- FTP (File Transfer Protocol)

- SSH (Secure Shell)

- DNS (Domain Name System)

- Which of the following is a network operating system?

- Windows 10

- Ubuntu

- macOS

- Windows Server

- Which type of network operating system allows multiple computers to access centrally stored shared resources and services?

- Peer-to-peer

- Client-server

- Standalone

- Mainframe

- Which network operating system is based on the Linux kernel and is widely used in server environments?

- Windows Server

- macOS Server

- Red Hat Enterprise Linux

- Ubuntu Desktop

- Lightweight Directory Access Protocol (LDAP) is a protocol used for:

- Email communication

- Database management

- Directory services

- Web browsing

- What does the acronym "OU" stand for in the context of directory services?

- Object Update

- Organizational Unit

- Operational User

- Outermost User

- Which directory service feature allows users to search for and locate specific information within the directory?

- Replication

- Caching

- Indexing

- Auditing

- Which protocol is used for automatic address configuration in IPv6?

- DNS (Domain Name System)

- ARP (Address Resolution Protocol)

- ICMPv6 (Internet Control Message Protocol version 6)

- NAT (Network Address Translation)

- Which of the following is an advantage of IPv6 over IPv4?

- Larger address space

- Better network performance

- More efficient routing

- Backward compatibility with older protocols

- What is the maximum number of unique IP addresses that can be assigned in IPv4?

- 2^16 (65,536)

- 2^32 (4,294,967,296)

- 2^64 (18,446,744,073,709,551,616)

- 2^128 (340,282,366,920,938,463,463,374,607,431,768,211,456)

- Which type of firewall operates at the network layer (Layer 3) of the OSI model?

- Packet-filtering firewall.

- Proxy firewall.

- Stateful inspection firewall.

- Application-level firewall.