Member ID: ____________________

Time: ___________________

Rank: _______________

Member ID: ____________________

Time: ___________________

Rank: _______________

Network Administration Using Cisco

(315)

State 2024

CONCEPT KNOWLEDGE:

Multiple Choice (50 @ 2 points each)	______________ (100 points)

Test Time: 60 minutes

Multiple Choice Questions

Directions: Identify the letter of the choice that best completes the statement or answers the question.

- What does the TCP protocol in the OSI model primarily manage?

- The physical transmission of the data

- The routing of data across different networks

- Ensuring reliable data transmission by managing data packet error checking and sequencing

- The direct communication between software applications

- What is the purpose of a VLAN in a network?

- It is used to redistribute routes from other routing protocols.

- It is a subset of a network that groups together devices from different physical LANs.

- It connects devices within a LAN through IP addressing.

- It is a firewall used to protect the network from external threats.

- What is the main function of EIGRP in network routing?

- It is used for local network management.

- It establishes secure VPN tunnels.

- It distributes IP addresses within a network.

- It is an advanced distance-vector routing protocol, used on a computer network for automating routing decisions and configuration.

- What is the primary role of a Router in a network?

- It translates IP addresses to domain names

- It provides the graphical user interface for network management

- It forwards data packets between computer networks

- It controls the access of computers to the network

- What is the primary role of a Dynamic Host Configuration Protocol (DHCP) server in a network?

- It distributes summarized routes of the area to other OSPF areas

- It translates domain names to IP addresses

- It assigns IP addresses to devices on a network

- It controls the access of computers to the network

- What is the function of the Simple Network Management Protocol (SNMP)?

- It provides a set of standards for network management, including monitoring and controlling network devices

- It is always the Designated Router of the OSPF area

- It always runs other routing protocols to translate them into the OSPF area

- It distributes summarized routes of the area to other OSPF areas

- What is the purpose of an Intrusion Detection System (IDS) in a network?

- It acts as a default gateway for the network

- It monitors network and/or system activities for malicious activities or policy violations

- It assigns IP addresses to devices on a network

- It translates IP addresses to domain names

- What does a Virtual Local Area Network (VLAN) do in a network?

- It encrypts all data traffic for enhanced security

- It physically separates devices into different geographic areas

- It logically separates devices on the same network into different subnetworks

- It distributes summarized routes of the area to other OSPF areas

- What is a purpose of Network Address Translation (NAT) in a network?

- It assigns IP addresses dynamically to devices on a network

- It manages the routing tables in an OSPF area

- It forwards data packets based on MAC addresses

- It converts private IP addresses into public IP addresses for outbound traffic

- Which interfaces would be impacted by the command "network 172.16.0.0 0.0.255.255 area 0" in OSPF?

- All interfaces on the router that have an IP address in the range 172.16.0.0 to 172.16.255.255

- All interfaces on the router that have an IP address outside the range 172.16.0.0 to 172.16.255.255

- All interfaces on the router that have an IP address in the form of x.x.x.0 (X indicates wildcards)

- All interfaces on the router that have an IP address in the form of x.x.x.255 (X indicates wildcards)

- Which command sets a Cisco switch interface to trunk mode?

- switchport mode trunk

- switchport trunk mode

- switchport set trunk

- trunk mode switchport

- How can you set the native VLAN for a trunk link to VLAN 50?

- switchport trunk native vlan 50

- switchport mode native vlan 50

- switchport native vlan 50

- native vlan 50

- Which command should you use to remove VLAN 10 from a Cisco switch?

- delete vlan 10

- remove vlan 10

- no vlan 10

- unset vlan 10

- What does the command "show ip route" do on a Cisco router?

- It shows the entire IP routing table.

- It shows only the active IP routes.

- It shows the MAC address table.

- It shows the routing protocols configured on the router.

- What is the result of running the command “show vlan brief” on a Cisco switch?

- It shows a brief summary of all VLANs and their associated interfaces.

- It shows the detailed configuration of all VLANs.

- It shows only the active VLANs.

- It shows the detailed information for a specific VLAN.

- What commands set and enable an IP Address on a FastEthernet interface?

- ip address 192.168.10.2 255.255.255.0, enable

- interface FastEthernet 0/1, ip address 192.168.10.2 255.255.255.0, no shutdown

- interface FastEthernet 0/1, address 192.168.10.2, enable

- FastEthernet address 192.168.10.2 255.255.255.0, no shutdown

- What command set is used to set up a static route to a particular network?

- ip route 192.168.10.0 255.255.255.0 192.168.1.1, enable

- ip route 192.168.10.0 255.255.255.0 192.168.1.1, no shutdown

- ip route 192.168.10.0 192.168.1.1, enable

- ip route 192.168.10.0 255.255.255.0 192.168.1.1

- How to set the clock rate on a Serial interface?

- interface Serial 0/0, clock rate 64000, enable

- interface Serial 0/0, clock rate 64000

- clock rate 64000, enable

- Serial 0/0, clock rate 64000

- How to set and enable Port Security on a FastEthernet interface?

- interface FastEthernet 0/1, switchport port-security, enable

- switchport port-security, enable

- interface FastEthernet 0/1, port-security, enable

- interface FastEthernet 0/1, switchport port-security, no shutdown

- Which of the following IP addresses is in the same IPv4 subnet as 192.168.16.24/27?

- 192.168.16.5

- 192.168.16.30

- 192.168.17.26

- 192.168.16.90

- Which of the following addresses belongs to the same IPv4 subnet as 172.16.10.64/26?

- 172.16.10.100

- 172.16.11.67

- 172.16.10.63

- 172.16.9.65

- Which of the following is a private Class A network address?

- 10.5.0.0/16

- 165.15.0.0/16

- 172.31.0.0/16

- 191.168.0.0/16

- What is the broadcast address for the IP 10.45.100.28/27?

- 10.45.100.31

- 10.45.100.59

- 10.45.100.63

- 10.45.100.127

- Your network uses the address range 10.0.0.0/8 and needs to be subdivided into at least 600 individual networks that will allow as many hosts as possible on each. Which subnet mask should you use?

- 255.255.128.0

- 255.255.248.0

- 255.255.240.0

- 255.255.192.0

- How would you verify the IPv6 configuration on a Cisco device?

- show ipv6 configuration

- show ipv6 interface

- ipv6 show routing

- show ipv6 route

- How can you disable IPv6 on a specific interface in a Cisco device?

- ipv6 disable

- no ipv6 enable

- no ipv6 address

- disable ipv6

- What is the primary function of the Cisco technology, Spanning Tree Protocol (STP)?

- STP is designed to prevent loop formation in a network.

- STP is used to create multiple virtual LANs on a single physical network.

- STP is a security protocol to prevent unauthorized access to a network.

- STP is a protocol to manage multiple gateways in a network.

- Identify the IPv6 Link-Local address:

- 2001:DB8::1

- 192.168.1.1

- FE80::1

- 172.16.0.1

- Which command is used to configure a default gateway on a Cisco switch?

- ip default-gateway

- ip route 0.0.0.0 0.0.0.0

- ip gateway default

- ip default-route

- A network administrator wants to minimize the time it takes for a port to transition from blocking to forwarding state in spanning-tree. Which of the following configurations should be implemented?

- Enable Rapid Spanning Tree Protocol (RSTP)

- Disable spanning-tree on the port

- Increase the port priority on the switch

- Configure the port as an access port

- A network administrator wants to decrease the convergence time of spanning-tree in the network. Which feature should be enabled?

- Bridge Protocol Data Units (BPDU) guard

- Root guard

- PortFast

- Loop guard

- The command "show interface serial0/0" was executed on a router, and the following entry is found: "Serial0/0 is up, line protocol is up". What can be determined from this output?

- The physical interface Serial0/0 is operational and connected.

- The line protocol on Serial0/0 is in the up state.

- The router is using the HDLC encapsulation protocol on Serial0/0.

- The router is experiencing errors on the physical layer of Serial0/0.

- When configuring VLAN Trunking Protocol (VTP) in a Cisco network, what is the purpose of the "transparent" mode?

- It forwards VTP advertisements but does not synchronize VLAN information

- It forwards VTP advertisements and synchronizes VLAN information

- It blocks VTP advertisements from being forwarded

- It allows VTP advertisements from unknown sources but notates the event by sending a message

- When configuring access control lists (ACLs), what is the purpose of the "deny" statement?

- It allows traffic matching the specified criteria

- It redirects traffic matching the specified criteria to another destination

- It blocks traffic matching the specified criteria

- It prioritizes traffic matching the specified criteria over other traffic

- In the context of Dynamic Host Configuration Protocol (DHCP), what is the purpose of the "exclusion" range?

- It blocks all incoming DHCP requests

- It allows incoming DHCP requests from unknown sources but notates the event by sending a message

- It prevents specific IP addresses from being assigned by the DHCP server

- It allows specific IP addresses to be assigned by the DHCP server

- Which command is used to configure a default gateway on a Cisco switch?

- switch default-gateway

- default-gateway ip

- gateway default

- ip default-gateway

- Which command is used to display the current configuration of a Cisco router?

- show running-config

- show startup-config

- show configuration

- show config

- Which command is used to verify the connectivity between two Cisco devices?

- traceroute

- ping

- show interfaces

- show cdp neighbors

- Which command is used to apply an access control list (ACL) to an interface?

- ip access-group

- ip access-list

- apply-acl

- apply-access-list

- In a Cisco ACL, which wildcard mask would match all IP addresses within a given network?

- 0.0.0.0

- 255.255.255.0

- 0.0.0.255

- 255.255.255.255

- Which of the following is true about wildcard masks used in access control lists (ACLs)?

- Wildcard masks are used to specify the source and destination IP addresses in an ACL.

- Wildcard masks are used to define the subnet mask in an ACL.

- Wildcard masks are used to identify which bits of an IP address should be ignored in an ACL.

- Wildcard masks are used to specify the action to be taken on packets matching an ACL.

- How can you verify the DHCP lease information for a specific client on a Cisco router?

- show dhcp server lease

- show dhcp binding

- show dhcp client lease

- show dhcp pool

- What is the default lease time for DHCP IP addresses?

- 1 hour

- 1 day

- 7 days

- 30 days

- What is the default port number used by DHCP?

- 20

- 53

- 67

- 80

- What is the recommended minimum number of NTP servers for reliable time synchronization?

- 1

- 2

- 3

- 4

- What is a firewall?

- A hardware device used to connect networks

- A software program that protects against unauthorized access

- A physical barrier preventing network communication

- A protocol used for secure file transfers

- What is the purpose of a VPN?

- To create a private network within a public network

- To increase internet speed

- To block unwanted advertisements

- To encrypt email communications

- What is the difference between HTTP and HTTPS?

- HTTPS is a more secure version of HTTP

- HTTP is used for websites, while HTTPS is used for emails

- HTTPS uses a different port number than HTTP

- HTTP is faster than HTTPS

- What is the purpose of an IP address?

- To identify a specific website on the internet

- To encrypt data for secure transmission

- To track the location of a mobile device

- To uniquely identify a device on a network

- Which protocol and port combination is commonly used for secure web browsing?

- UDP port 80

- TCP port 443

- UDP port 443

- TCP port 80